CMPG 325-COMPUTER NETWORKS

LAMOLA MC

40624102

CLIENT REQUIREMENTS

CLIENT INFORMATION

| Item | Details |
| --- | --- |
| Client ID | CLI-028 |
| Organization | Ipeleng Study & Tutoring Hub |
| Location | Kimberley |
| Industry | Education |
| Addressing Block | 172.30.10.0/23 |
| Networking Challenge | VLANs (switch-based segmentation) |
| Design Constraint | Shared risers – no civil works allowed |
| Change Request | CR5 – 25% user growth, no renumbering |

CLIENT

Ipeleng Study & Tutoring Hub is a tutoring centre located in Kimberley that supports learners in their academic development. Its primary users include students attending tutoring sessions, staff members, and occasional visitors such as parents who come to inquire about the services offered.

CLIENT MAIN REQUIREMENTS:

- Separate networks

They want to put staff, students and visitors on different networks to prevent students from seeing staff information and have guests only have access to the internet.

- VLANs

They want VLANs to separate the traffic. VLANs are a good choice because they let me split the network logically without buying extra hardware.

- No Civil Works

They rent part of the building, so they can’t drill into the walls or install new cables. I’ll have to work with the existing risers and use the cable pathways that are already in place.

- Future growth

They’re expecting about 25% more users, so the network needs to be able to handle the extra demand without requiring any changes to the existing IP addresses or renumbering the network.

HOW TO MEET THEIR REQUIREMENTS

| Requirement | My Solution |
| --- | --- |
| Separate networks | Use VLAN 10 (Staff), VLAN 20 (Students), VLAN 30 (Guests) |
| VLANs | Set up VLANs on switches and configure inter-VLAN routing on the router |
| No civil works | Use a star topology with all switches in one central location |
| 25% growth | Choose subnets with spare IPs available |
| No renumbering | Plan addressing so there are enough extra IPs from the beginning |

2. PHYSICAL TOPOLOGY

My Physical Design

I decided to use a star topology for this network. This means all switches connect back to one central switch. Here is my design:

Devices I Will Use

| Device | Model | Purpose |
| --- | --- | --- |
| Router | Cisco 1941 | Connects to internet and does inter-VLAN routing |
| Core Switch | Cisco 2960 | Central switch connecting all access switches |
| SW-1 (Staff) | Cisco 2960 | Connects staff PCs and printer |
| SW-2 (Students) | Cisco 2960 | Connects student workstations |
| SW-3 (Guests) | Cisco 2960 | Connects guest Wi-Fi access point |

Why I Chose This Design

| Reason | Explanation |
| --- | --- |
| Star topology | Easy to manage – all switches connect to one core switch |
| Minimal cabling | Uses existing risers – no new cables through walls |
| No civil works | All equipment fits in the existing network closet |
| Scalable | Easy to add more switches or devices later |
| Central management | All traffic goes through the core switch – easy to monitor |

Where Will All the Equipment Be Located?

All equipment will be located at a network closet (Telecom Room). The existing closet already has a number of cable risers that can be used to provide cabling to their respective destination areas.

| Device | Location |
| --- | --- |
| Router | Telecom room |
| Core Switch | Telecom room |
| SW-1 (Staff) | Telecom room |
| SW-2 (Students) | Telecom room |
| SW-3 (Guests) | Telecom room |
| Staff PCs | Staff office (connected via existing wall ports) |
| Student PCs | Study area (connected via existing wall ports) |
| Wi-Fi AP | Guest area (connected via existing wall port) |

Cabling Plan

| Connection | Cable Type | Notes |
| --- | --- | --- |
| Router to Core Switch | Copper (Cat6) | Short cable – same rack |
| Core to SW-1 | Copper (Cat6) | Uses existing riser |
| Core to SW-2 | Copper (Cat6) | Uses existing riser |
| Core to SW-3 | Copper (Cat6) | Uses existing riser |
| SW-1 to Staff PCs | Copper (Cat6) | Existing wall ports |
| SW-2 to Student PCs | Copper (Cat6) | Existing wall ports |
| SW-3 to Wi-Fi AP | Copper (Cat6) | Existing wall port |

How This Meets the Constraints

| Constraint | How I Meet It |
| --- | --- |
| No civil works | No drilling or wall modifications – using existing risers |
| Shared risers | All cables run through existing pathways |
| Rented building | All equipment stays in the telecom room – no changes to the building |

3. Logical Topology

| VLAN | Name | Subnet | What It's For |
| --- | --- | --- | --- |
| 10 | Staff | 172.30.10.0/25 | Staff PCs and printer |
| 20 | Students | 172.30.10.128/25 | Student computers |
| 30 | Guests | 172.30.11.0/26 | Guest Wi-Fi |
| 99 | Management | 172.30.11.64/27 | Switches and router management |

WHY I SET UP LIKE THIS:

- Staff (VLAN 10): Needs access to the printer and files. This VLAN is separated from students for security reasons.

- Students (VLAN 20): Require Internet access, but they should be denied access to staff files.

- Guests (VLAN 30): Have access to a limited Internet connection. They should not have access to the organization’s network resources.

- Management (VLAN 99): For switches only. It should be isolated from other VLANs for security reasons.

LOGICAL DIAGRAM

COMMUNICATION BETWEEN THE DEVICES

| Traffic Type | How It Works |
| --- | --- |
| Within same VLAN | Devices communicate through the switch – no router needed |
| Between different VLANs | Traffic goes through the router (inter-VLAN routing) |
| To internet | Traffic goes through the router to the internet |
| Management | Network admins access switches via VLAN 99 |

Inter-VLAN Routing

- I will configure the router using the method called the Router-on-a-Stick, which consists of the following:

•Establish a trunk link from the core switch to the router

•Set up the router with several sub-interfaces, each VLAN having one sub-interface

•Assign an IP address to each of the sub-interfaces which will be the default gateway of the VLAN it is assigned to.

| VLAN | Sub-interface | Gateway IP |
| --- | --- | --- |
| 10 | Gig0/0.10 | 172.30.10.1 |
| 20 | Gig0/0.20 | 172.30.10.129 |
| 30 | Gig0/0.30 | 172.30.11.1 |
| 99 | Gig0/0.99 | 172.30.11.65 |

How This Meets the Requirements

| Requirement | How VLANs Meet It |
| --- | --- |
| Separate networks | Each user group has its own VLAN |
| Security | VLANs isolate traffic – no cross-VLAN access without routing |
| Performance | Broadcast traffic stays within each VLAN |
| Organisation | Easy to manage and troubleshoot |

4.  IP Addressing plan

IP Addressing Table

| VLAN | Name | Subnet | Subnet Mask | Usable IPs | Gateway |
| --- | --- | --- | --- | --- | --- |
| 10 | Staff | 172.30.10.0/25 | 255.255.255.128 | 126 | 172.30.10.1 |
| 20 | Students | 172.30.10.128/25 | 255.255.255.128 | 126 | 172.30.10.129 |
| 30 | Guests | 172.30.11.0/26 | 255.255.255.192 | 62 | 172.30.11.1 |
| 99 | Management | 172.30.11.64/27 | 255.255.255.224 | 30 | 172.30.11.65 |

Static IP Addresses

| Device | IP Address | VLAN |
| --- | --- | --- |
| Router (VLAN 10 gateway) | 172.30.10.1 | 10 |
| Router (VLAN 20 gateway) | 172.30.10.129 | 20 |
| Router (VLAN 30 gateway) | 172.30.11.1 | 30 |
| Router (VLAN 99 gateway) | 172.30.11.65 | 99 |
| Core Switch | 172.30.11.66 | 99 |
| Staff Switch | 172.30.11.67 | 99 |
| Student Switch | 172.30.11.68 | 99 |
| Guest Switch | 172.30.11.69 | 99 |
| Staff Printer | 172.30.10.5 | 10 |

DHCP Ranges

| VLAN | DHCP Range |
| --- | --- |
| Staff (VLAN 10) | 172.30.10.10 – 172.30.10.126 |
| Students (VLAN 20) | 172.30.10.140 – 172.30.10.254 |
| Guests (VLAN 30) | 172.30.11.10 – 172.30.11.62 |
| Management (VLAN 99) | No DHCP – static only |

How I Handled CR5 (25% Growth)

| Item | Value |
| --- | --- |
| Total IPs available | 512 |
| IPs I am using | 344 |
| Spare IPs | 168 |
| 25% growth (86 users) | 168 spare IPs is enough |
| Need to renumber? | No |